"TECHNICAL SPECIFICATION FOR EVALUATION"

S.T.V. 9991332

EQUIPMENT SUPPORT SYSTEM (SKID), OF AN FRU TARTARINI REDUCTION LINE

ND 25-40-50-65-80-100

| Issuing Department SERTEC-NORM | Compiled by LORO, SILVIO | Checked by VARESE, ENRICO | Approved by COMAZZI, MARCO |
| --- | --- | --- | --- |

Table of contents

1	PURPOSE	3

2	CHARACTERISTICS	3

3	USE	5

4	SUPPLIES	5

# PURPOSE

This specification illustrates the support system (SKID) of the equipment constituting an FRU or IRP type reduction line, manufactured by TARTARINI.

Below is photographic documentation of one of these skids, indicating the positioning and installation.

FRU Tartarini ND 40 double line

# CHARACTERISTICS

The construction characteristics of these skids are such that they can be adapted to the configuration of TARTARINI FRU and IRP type systems and are sized according to the total weight of the assembled unit consisting of the BM5+OS/80 blocking device with the MFL-BP compact reducers, for the following nominal diameters: ND 25-40-50-65-80-100.

The frame is made of welded steel profiles, painted to prevent corrosion.

Installation and fastening are carried out with the help of stainless steel collars and screws, to be positioned:

- upstream, on the coupling flanges of the BM5 blocking device;

- downstream, on the horizontal outlet manifold of the reduction line, immediately after the coupling flange of the MFL reducer spacer with the manifold itself.

Outline construction details

# USE

The skid supports the aforementioned equipment, installed on the specific line, during the disassembly step for carrying out maintenance activities and replacement of internal component parts.

The criterion adopted for its installation and fastening allows easy disassembly, transport and reinstallation of the skid on the various lines present on the TARTARINI FRU and IRP type plants, with the same nominal diameter ND of the equipment.

# SUPPLIES

The supply of each individual skid consists, for each ND of the equipment installed on the FRU or IRP type line of the plant manufactured by TARTARINI, of:

- no. 1 frame;

- no. 2 stainless steel collars, fitted with screws, washers and fixing nuts;

- no. 1 document containing:

- the installation and operating instructions;

- indication of the type of line (ND) on which it can be used;

- the maximum sustainable weight, understood as the maximum total weight of the assembly

consisting of the combination of the BM5+OS/80 blocking device with the MFL-BP compact reducers.

Henceforth referred to as SKID